# General Transit Feed Specification (GTFS) Static Data Creation & Maintenance

Contents

General Transit Feed Specification (GTFS) Static Data Creation & Maintenance	1

Using the SOP	3

Referencing Data	3

Quickstart Guide	4

High-Level Workflow	4

Utilities	5

Unintended Data Edit Effects	5

Setting Service Types	5

Using Patterns to Create Stop Times	5

What is GTFS?	7

GTFS Overview	7

GTFS Specification	7

How GTFS Works	8

Required Files	9

Optional Data	12

Files Required for GTFS Creation Workbook	12

Examples of Other Optional Files	13

Understanding Lookups	14

Building a GTFS feed	17

Spreadsheet Contents	17

Representing Deviated Fixed-Route Service	31

Editing a Feed	34

Importing an Existing Feed	34

Routine Data Updates	34

Adding or Deleting Routes	35

Adding or Deleting Stops	35

Editing Calendar	35

Editing Trips and Patterns	37

Clearing Stop Times	37

Adding or Replacing a Shape	37

Feed Validation and Debugging	39

Hosting a GTFS Feed	42

A. Appendix	44

Data Validation and Restrictions	44

Help Text	44

Data restrictions and dropdowns	44

Worksheet Protections	44

Excel Tips	45

Find and Replace	45

Creating Shapes in Google Earth	48

Creating Shapes in Google MyMaps	51

Hosting Files on Google Drive	55

# Using the SOP

This SOP is intended to cover how to create and use GTFS data from start to end; as such, it’s pretty long! You may be reading this because you need to make a schedule change in the very near term. If you need to get started making updates to the workbook to make a functional feed, you can jump directly to Building a GTFS feed, or consult the Quickstart Guide to quickly get into GTFS editing. Beginners to GTFS should proceed directly to the What is GTFS? section to gain some background on the GTFS file structure.

## Referencing Data

This document often refers to worksheets in the data creation workbook, GTFS files, and columns within either the workbook or GTFS files. Some text formatting is used to refer to each to make understanding which is which consistent to follow:

Sheets or utilities in the workbook: bold.

Ex: Agency.

GTFS file names: italicized, with .txt at the end.

Ex: agency.txt.

Columns in either the workbook or individual GTFS files:  monospace font and red.

Ex: agency_id.

# Quickstart Guide

## High-Level Workflow

The workbook contains a number of data validation rules and hints for working with the GTFS data. As such, if you need to make any edits it is strongly recommended to review any context hints on each column, and work from macro-to-micro scale of data. GTFS data is highly relational – each table in the workbook relates to one or more others, which is used to represent your schedule data. It’s useful to keep a general hierarchy in mind when creating your feed:

Agency

Calendar / Calendar Dates

Routes

Trips

Stops

Stop Times

Shapes

When you are making edits to your data, it’s helpful to consider the impacts. If you are adding a new route, for example, you would use a workflow such as:

- Add the route to the Routes worksheet.

Determine how it operates relative to your existing service_ids in the Calendar worksheet.

Is it a special service such as for festivals or a shopping shuttle that might only operate on specific days? You may need to add a new Service type to your Calendar.

Add the Trips that it will operate.

This is a good time to consider the Patterns that it may use.

Patterns is not part of the GTFS spec but is included in this workbook to facilitate adding your Stop Times which can be very cumbersome to manage.

Add any new Stops that this route needs to serve.

Any existing stops on other routes do not need to be added to your Stops table, but new stop data would need to be determined such as the stop_id, stop_name, and latitude/longitude.

Create your Stop Times.

You can do this manually, or by using the Patterns functionality in the workbook.

Create your Shapes.

Shapes is a bit of a chicken or egg scenario – you need to create your shapes and assign them to your Trips, but it can be difficult to determine the appropriate shape until you have your stops identified.

Since this is more iterative, there aren’t as many guardrails for validation as compared to other data in the workbook.

## Utilities

The workbook contains a number of VBA based utilities in the Utilities worksheet. Each utility is initiated with a button, and utilities that modify or process data generally require the use to confirm the operation. Refer to Automation utilities for more information.

## Unintended Data Edit Effects

The previous section had a long-winded way of running through all the data you would need to update when adding a route. Adding a route is a pretty big deal, so there is a lot to consider.

Smaller edits can be made quickly, but it is important to consider the relationship between your data and any cascading effects. For example, if you are skipping a stop on one or two trips, you may be tempted to simply remove the stop from your patterns and recreate your stop times. Doing it that way, however, would lead to the stop being removed on all trips. You instead have a few options:

- Create a new pattern to reflect the variation.

- Find the specific trips in your Stop Times and manually remove the stop.

- Update the pickup/drop_off rule in Stop Times to reflect the stop being skipped.

It’s worth considering what is unique within GTFS, and what’s specific to an individual trip. The relationships between each table are critical, and it’s easy to break a feed if you don’t consider how tables relate.

## Setting Service Types

Service_id is one of the easiest columns to misuse, but foundational to how your feed works. A service is when and how your buses run. Services are generally defined in the Calendar workbook for each day of the week. A “0” means the service does not run on that day, while a “1” indicates it does. So, if you would want to setup Weekday service, you would simply assign monday through friday with 1’s, and saturday/sunday with 0. This is then reflected with a service_id, which is tagged to trips.

Calendar Dates is typically used for service exceptions. This would be adding or removing service on a specific day. For example, if you run a reduced schedule on holidays, you would assign an exception_type that removes the service in use that day, and then adds the reduced schedule.

The GTFS Creation Workbook includes hints within each column header to identify what these unique values mean, along with the detailed GTFS creation process in Building a GTFS feed.

## Using Patterns to Create Stop Times

Patterns is the main departure from the GTFS standard in the workbook. The workbook keeps things simple, which generally means sticking to the standard in a barebones manner.

Patterns in the workbook refers to any unique stop sequence of running times for a route and direction. Patterns is purely optional – you could ignore it entirely, and in some cases, it may make sense to. To create a pattern, you’d simply create a pattern_id, set the sequence of stops it serves, and then assign the minutes from the start of each trip that the vehicle arrives and departs at the stop.

This is then combined with optional fields in the Trips table to create Stop Times. After you have created your patterns, you go back to your Trips table and assign each trip to a pattern, along with the trip start time. Then just click the Create Stop Times button under Utilities.

The Stop Times table can get very large very fast, which is why manually generating the data is cumbersome. For example, if you have 20 stops per trip and 20 trips, Stop Times needs to reflect every stop time on each trip, meaning you have 400 records to manage. Patterns simplifies this by focusing on the unique variations assigned to types of trips.

It may not make sense to use patterns for highly variable routes. You can use a mix of manually generated Stop Times data and automatically generated, as the Create Stop Times utility will add data to the end of your existing Stop Times.

# What is GTFS?

## GTFS Overview

GTFS, or General Transit Feed Specification is a standardized format for transit schedule data. Transit schedule data comes in several formats, from as simple as an Excel file, to complex and proprietary standards often used by advanced scheduling software. GTFS serves a common standard specification for fixed-route transit schedule elements. This allows you to easily communicate your schedule data to interested parties, or even import into different software for ease of setup and analysis.

A standard GTFS static feed is a zipped set of text files. Text files are not in any parent directory, but instead directly zipped. The name of the GTFS zip itself does not matter, as long as there are not forbidden characters based on your file system.

GTFS consists of several required and optional files and columns. Required files or columns are core to the functionality and relationship between schedule attributes, and the GTFS data set is invalid without them. Optional files add more data and context to your schedule - such as identifying the vehicle block that operates a trip – but are not required for a GTFS to communicate your schedule. Optional files may be needed for certain use cases, such as more detailed understanding of your schedule or to communicate deviated fixed-route standards. GTFS component files, such as your routes.txt that describes your routes, have mandatory names and data requirements.

The next section will briefly describe the required files in a GTFS static feed, and their usage. The full GTFS specification is available at Mobility Data’s website: https://gtfs.org/schedule/reference/.

## GTFS Specification

The specification is maintained by MobilityData, and detailed information is available at the GTFS intro page [https://gtfs.org/]. This specification consists of a core static schedule specification (currently known as “Schedule” [https://gtfs.org/schedule/]), and several optional extensions. This document focuses on utilizing the core GTFS static Schedule specification to create deviated fixed-route feeds.

GTFS static – the specification used in this document, static schedule data in a .zip file.

GTFS Real-Time – streaming real-time updates pertaining to a static GTFS schedule feed.

Within the GTFS static specification there are also optional extensions. These extensions provide enhanced functionality; however, they have limited support and can be difficult to develop or maintain. For example, the optional GTFS Flex specification can represent more detailed flexible, zone-based, or route deviation-based services, however it is experimental, challenging to implement/maintain, and has limited support among trip planning providers. Transit agencies must create zone-based shapefile data in the GeoJSON file format, integrate this data into their GTFS feed using relational IDs, and ensure that the data is kept up to date. Few data consumers can make use of the GTFS Flex data at this time and will ignore this data in most cases.

## How GTFS Works

GTFS static data is a single zip file containing several plaintext files with the .txt file extension. These files are human readable (meaning you can read the text) but can be difficult to follow due to relationships between each file. The individual GTFS component files provide a way to communicate an agency’s scheduled service. The files do this by functioning as a portable database. Each GTFS file relates to others using a common lookup field, or column. This field functions like a primary key in a database, or a lookup value if you are coming from Excel. Provided the common value is present in both files, you can connect and retrieve the data as needed.

GTFS files are essentially database tables, similar to Excel workbooks. There are headers at the top of each file to identify what the column is (such as route_short_name to store the route name), and then data below it. Data is stored as Comma Separated Values (CSV) in a plaintext file format. CSVs are a way to store tabular, or table-based data in a simple way.

An important concept to keep in mind when working with GTFS data is uniqueness. Every record in every file/worksheet is unique in some way, which allows it to be referenced within the specification. For example:

There is only one Route A, which is stored in the routes.txt file. Trips operated by Route A have their own ID, which are stored in the trips.txt file. This then allows you to connect the routes.txt file to the trips.txt file to retrieve the route name or to count the number of trips operated by Route A.

Times that stops are serviced by Route A on each trip have their own unique records in stop_times.txt to help ensure that they are not duplicated.

This uniqueness, and connection between aspects of your GTFS data, are critical to packaging your schedule data.

GTFS Relationships Example

Assume that you operate Route A in your schedule. This route would have an internal GTFS ID, which may or may not match your route numbering/naming schema. In our example, this is identified as route_id=1. The route_id key field in routes.txt is then used to look up trips operated by this route within trips.txt as both files have a common route_id value. ID values, such as 1, A, or even a short bit of text, are useful identifiers. Very terse values such as 1/2/3 can be difficult to follow, but depending on your GTFS feed can be much more efficient to store. These relationships are used to draw on a major benefit of a database structure: avoiding the duplicative representation of data. When you want to look up trip information about Route A, you don’t want to store or process all of the other information about Route A, such as the branding colors, long name, or other route information. By using these keys to connect data together you are able to avoid representing this unrelated data multiple times and can instead have a very efficient data structure that has a much more compact file size and can be much faster to work with (for computers, though not necessarily people).

## Required Files

The list of required files below is meant to introduce specific files that are needed for a minimal GTFS static feed. These files are detailed further later in this document, including the specific data types required.

#### Agency.txt

- The Agency.txt file (Figure 1) is purely about your Agency – it consists of information the agency name, GTFS internal ID, contact information, and time zone.

- The agency file is often overlooked but is foundational to your GTFS feed. Many agencies neglect to set the agency_id field, which can cause the feed to be invalid and rejected. Other fields, such as time zone, are required to understand your feed – simply setting a stop arrival time of 7 AM is not much use if the GTFS processor isn’t able to determine what time 7 AM is relative to where the feed is processed.

- Time zone names come from the Internet Assigned Numbers Authority, though Wikipedia currently has an updated list that reflects acceptable timezone identifiers. If you are in Ohio, for example, you would likely use America/New_York.

Figure 1: Agency.txt example

Note: in .txt files, the first row represents column headers which are separated by commas. Rows of data start on the second line.

#### Calendar.txt

- Calendar.txt (Figure 2) lists the service days of the week along with the overall start and end dates. It uses '1' to indicate the service is available for that day in the date range and '0' to indicate it's not available. Each day of the week is included in its own column, such as Monday, Tuesday, and so on through Sunday.

- Service is defined as a unique service_id. Your service_id could be whatever makes sense for your agency, Weekday/Weekend, M-Th/Fri/Sat/Sun or 1/2/3. Whatever service_id you decide to use, it must be consistent across your feed.

- Note that you can use service IDs to reflect different service types, or service changes. For example, a Bus-Weekday could operate the same days and date ranges as a Trolley-Weekday. You can also create service types with disjoint date ranges, such as 12/1/2023 – 1/1/2023 and 1/2/2023 – 4/1/2023. This can be used to reflect a service change going into effect, though you will need to duplicate all unchanged service in your GTFS data between the different service IDs.

Figure 2: Calendar.txt example

#### Calendar_dates.txt

This file works in conjunction with calendar.txt (Figure 3) to define exceptions to your regularly scheduled service. This would typically be holidays; and allows for the explicit activation or deactivation of a service. It is possible to use solely calendar.txt or calendar_dates.txt, however defining consistent date ranges in calendar.txt, and service exceptions as calendar_dates.txt records may be easier to understand and maintain.

For example, your agency may operate Sunday level service on some holidays. You would use this file to indicate that your regular service is removed for the specific holiday date on one line, and then another line to indicate that Sunday service is being added for the same date.

Figure 3: Calendar_dates.txt example

#### Routes.txt

This file lists the routes operated by the agency including their ID, name, and branding colors (Figure 4). Agency ID is conditionally required here as some GTFS feeds may contain multiple agencies; if your agency is the only agency in the GTFS feed, you can omit the agency_id column from routes.txt. This file does not include the route’s time of operation or operating statistics; that information is present in the stop_times.txt table.

Figure 4: Routes.txt example

#### Trips.txt

This file contains the trips operated by the agency (Figure 5). Trips information includes the service_id that the trip operates under (from your calendar.txt, such as Weekday/Weekend), the route that operates the trip, trip headsign, and the route alignment used (see Shapes.txt). This file only describes what the trip is, not the time it operates.

Figure 5: Trips.txt example

#### Stops.txt

This file includes all stops serviced by the agency including their unique and code, name, and coordinates (latitude/longitude). This is a unique list of all your stops (see Figure 6).

Figure 6: Stops.txt example

#### Stop_times.txt

This is the file that indicates when your service operates (Figure 7). This file details the time and sequence that stops on all trips are served, including the trip ID, stop sequence, and then arrival and departure times. This is often the largest file in a GTFS feed. Every trip on every service ID is represented at the stop level.

For example, consider a Route A operates Weekday, Saturday, and Sunday service with a 7 AM trip. This 7 AM trip has the same times on all service days and serves the same 20 stops. This would then require 20 stop times for the Weekday trip, 20 stop times for the Saturday trip, and 20 stop times for the Sunday trip. This file can become very difficult to work with very quickly and is often a source of data quality issues.

Figure 7: Stop_times.txt example

## Optional Data

There are several optional files and columns in GTFS. This section describes a few of these that can add additional data to provide enhanced information to your customers.

Some GTFS files are technically optional but are required for the GTFS Creation Workbook. This section details these two specific optional files and provides additional examples of other optional files.

### Files Required for GTFS Creation Workbook

#### Feed_info.txt

This file details information about your feed such as the overall start and end date, and feed version (

Figure 8). This is included in our GTFS workbook – while optional, it makes processing your GTFS data substantially easier and can help identify errors in schedule periods.

The feed_start_date and feed_end_date in feed_info.txt do not necessarily need to align with your calendar.txt file. For example, you may have multiple weekday service_id date ranges in your calendar.txt file to reflect the GTFS feed covering two service changes. In such a case, your date range in the feed_info.txt would cover the total date range between your two different weekday service changes. Providing the total date range then helps GTFS feed consumers better process your feed and can help with analyzing or working with your GTFS data.

Figure 8: Feed_info.txt

#### Shapes.txt

Shapes.txt is an optional file that plots a trip’s path using latitude, longitude pairs and a sequence (Figure 9). Shapes are assigned a shape_id that are assigned to trips. This is technically an optional file in the GTFS standard but is considered mandatory for the purposes of deviated fixed-route service as it is required to identify flag-stops or other route deviations. It also allows for route lines to be visualized in GIS and other spatial programs.

Figure 9: Shapes.txt

### Examples of Other Optional Files

#### Fares.txt

This file lists fares by the service you operate, allowing trip planners such as Google Maps to show your riders an estimated fare for their trips. This is not included in the GTFS Workbook.

#### Transfers.txt

Trip planners such as Google Maps can calculate some transfers; however, this is computationally intensive and may not perfectly reflect your service. Explicitly identifying from and to transfers by trip in transfers.txt can provide you with better control over trip planners and how they connect riders with your service.

## Understanding Lookups

Lookups, also called joins, are critical to how your GTFS static file works and is used. Lookups use values in each table so the table can be joined and combined with other tables. Broken or invalid lookups will make your feed invalid and trip planners such as Google Maps may be unable to process your feed. This section describes several important caveats to consider when building lookups. Table 1 summarizes key join columns that exist in GTFS and how they relate to other tables.

Table 1: GTFS Relationships

| Table A – value is unique In this table | Connects To Table B – multiple values |
| --- | --- |
| Agency: agency_id | Routes: agency_id |
| Routes: route_id | Trips: route_id |
| Trips: trip_id | Stop_Times: trip_id and stop_sequence. (exception to the rule: uniqueness from shape_id and shape sequence order) |
| Stops: stop_id | Stop_Times: stop_id |
| Shapes: shape_id and shape_pt_sequence (exception to the rule: uniqueness from shape_id and shape sequence order) | Trips: shape_id |

Data type/structure matters

If your data is structured as text in one file and a number in another, the lookup will fail. Computers store data in a variety of formats, and numbers and text are two storage types. They may look very similar, but the number 2 is not the same thing as “2” stored as text.

This is particularly difficult to detect when using Excel. Excel is easy to use and automatically handles many transformations for you, but the software will at times perform transformations you do not anticipate. As such, it is best to do all of your data work in a single platform and consider how you are representing your data.

For example, ID numbers should be consistently formatted. An ID of 99 between files should be the number 99 in both files, rather than padding with characters (such as “099”), as Excel may truncate 0s. Keep an eye out for notifications in Excel that numbers are “formatted as text” or other warnings. These can often indicate that data is not behaving in the manner you are expecting.

#### Unique IDs

Each table contains an ID that is typically unique to that table. There is an exception here for shapes.txt, and stop_times.txt, though these files have a sequence column that creates uniqueness, as each ID has a unique sequence value (the stop_sequence is unique for each trip_id in the stop_times.txt).

It is important to ensure that the unique ID field in each table remains unique. For example, in routes.txt, routes are identified by a route_id, in addition to more familiar values like route_short_name and colors. You can control what route_id is set as, but it must be unique to that route. If there are duplicate route_ids, even if every other column for those routes is the same, you have violated the column constraints and will end up with an invalid GTFS feed.

#### Join Types

Joins come in several key types that are described in the sections below. Table 2 has brief examples of each join type, with longer descriptions after. Most relationships in your GTFS data will be One-to-Many, meaning that your higher-level data will relate to multiple lower-level records (such as one agency operating multiple routes).

Table 2: Join Type Summary

| Join Type | Description | Example |
| --- | --- | --- |
| One-to-One | One record in Table A connects to one or none records in Table B. | A rider ID corresponds to a single passenger’s information. |
| One-to-Many | One record in Table A can relate to one, none, or many records in Table B. | One route can operate many trips. |
| Many-to-Many | Many records in Table A can related to one, none, or many records in Table B. | Many passengers may qualify for multiple special fare programs. |

##### One-to-one

In a one-to-one join, one record in one table relates to exactly one other record in another. There are not many examples of this in GTFS, however, unless you had a single route that only operated a single trip.

##### One-to-many

In a one-to-many join, one record can relate to several other records, or vice-versa. For example, you only have a single Route A in routes.txt. This route, however, operates many different trips. So, any time you connect your route information (such as the route name) to the trips operated by the route, you have a one-to-many join. This is the most common relationship in our GTFS feed, as it benefits from separating the data into individual tables/files.

This is a very common relationship and can be broken by having duplicative route_id values. If multiple routes in routes.txt use the same route_id, you end up with an invalid join type.

##### Many-to-many (Error joins)

Many-to-many joins mean that several records from one table join to several records from another. This may be an undesirable behavior and is the result of not setting IDs correctly between records. For example, having duplicate trip IDs in trips.txt, that have different shape_ids in their shapes.txt.

Many-to-many joins often expand your data substantially, which may indicate an error or unanticipated relationship. Ensuring that you are using unique IDs in each file (route_id in routes.txt, trip_id in trips.txt, stop_id in stops.txt, and so on) will help you avoid many-to-many joins and ensure that your join relationship works as expected.

#### Other Lookup Issues

GTFS is a simple data structure. Each file is one aspect of your agency and uses one or more unique ID fields to connect files together. That said, as you edit, update, and maintain your GTFS feed, you may run into some of the issues described in this section.

##### Invalid ID Key relationship

In this case, a unique ID may exist, or be in use in one file, but not present in another. For example, you may have a trip that refers to a service ID (such as Sunday) that does not exist in your list of service IDs. These can be fixed by either correcting the service ID to one that does exist, or adding the service ID, as is appropriate.

##### Failures to Join

You may have a relationship that looks like it should work but fails to join in a validator. For example, a Route 10 may exist in both your routes.txt file, and have trips associated with it in your trips.txt file. In these cases, there may be a data type issue. Excel will often attempt to handle data types for you, but it may be that Route 10 is in one file as a number (such as 10), while in another file as text or padded with other characters (“10”, or “010”). These can be fixed by setting everything to Text, or setting everything to Number, as makes sense for your agency. If you have a mix of route numbers, letters, and text-based names, keeping everything as Text would be necessary.

# Building a GTFS feed

The prior sections delve into database designs and constraints, which can make GTFS feeds seem overwhelming. Ultimately, GTFS feeds are built entirely out of things your agency already knows, the issue is transforming this data into the correct structure.

You already know your:

Route names

Route branding

Route alignments/directions

Stop names and locations

Timetables

You can easily begin building your GTFS feed using the accompanying GTFS Creation Workbook.xlsm. (Figure 10) This workbook consists of several worksheets that are essentially just your schedule data, and came preloaded with your active schedule at the time of data submission. From there, you can update as needed and easily remove extraneous data. Each worksheet in the workbook is described below, along with the required and optional fields.

Figure 10: GTFS Creation Workbook

### Spreadsheet Contents

The spreadsheet tool allows you to enter information in a somewhat simplified format. There are a few utilities included that help to manage, transform, and export your data as needed. This section will identify how to spell out all data in your feed.

You should save your GTFS feed status often, particularly with descriptive file names. Multiple versions of the file can be saved, giving you points to revert to or support alternative working session if you are testing the impact of different schedule adjustments.

There are also several validation rules, help texts, and locked components of the spreadsheet. For more information on each, refer to Data Validation and Restrictions in the Appendix. Help Text for each column can be accessed by clicking the column header within any data entry worksheet.

The GTFS Creation Workbook consists of several individual worksheets.

- Instructions – details on how to use the workbook.

Utilities – automation utilities to facilitate GTFS creation – each utility has a description of how it works, in addition to the descriptions later in this document.

Agency – GTFS agency data.

Routes – GTFS routes data.

Trips – GTFS Trips data.

Stops– GTFS Stops.

Patterns – This is an optional table specific to the workbook that does not have a representation in GTFS. This table is configured for use with one of the utilities to allow much easier creation of Stop Time.

Stop Times – GTFS stop times data.

Shapes – GTFS shape data to allow better representation of the route’s alignment.

#### Instructions

This worksheet houses some instructions and tips for working with the workbook.  This also includes a legend for which fields are required, optional, or required but typically will be a single value such as your agency_id or the route_type. An example of the legend from the Instructions sheet is available in Figure 11.

Figure 11: Legend from Instructions Sheet

#### Agency

Agency contains simple information about your agency. This information includes your agency name, website, contact phone number, customer service email, time zone, and language. Your agency information is unlikely to change often, but this file is foundational to your GTFS feed.

agency_id – Required, a short identifier for your agency.

agency_name – Required, the proper name for your agency, either as an acronym or fully written out.

agency_url – Required, a URL to find your agency’s schedule and contact information.

agency_language – The language code for your agency (generally en for English or es for Spanish), used by some trip planners to localize your service information.

agency_timezone – Required, the time zone for your agency, used to determine your trip times within the trip planner.

agency_email – Optional but recommended, a customer facing contact email for your agency.

In this tab, just enter the information pertaining to your agency. This data describes how your agency can be contacted for customer service or other inquires. An example agency data entry is displayed in Figure 12.

Figure 12: Agency Worksheet

#### Calendar

Calendar identifies the specific service days and types operated by your agency. General service days are specified as weekdays, such as 'Monday' or 'Saturday', followed by a start and end service date in the format of year-month-day (e.g., 20240101), indicating the operational date range. You may have multiple service types operating on a weekday, such as if you operate fixed-route bus service and trolley service.

Dates are identified by the date start range, date end range, and then a series of 0’s (for no service) or 1’s (that service operates) for each day of the week. For instance, you will see 1 under Monday if service is available for all Mondays within your specified start and end range, and 0 if service is not available, as shown in Figure 13.

Figure 13: Calendar Worksheet

Service types can be difficult to fully grasp – there is no file that spells out what a service is. Services are applied to a date range, such as Weekday service operating Monday through Friday, and then the date range the service is active for. Service IDs are then added to trips to indicate that they operate Weekday, or other service types. Each Service ID should be unique and will be referenced in trips.txt file.

Your own familiarity with your agency can help you determine how to best structure your service. Thinking over your schedule, try to identify the most simple representation of your service. If your agency operates Weekday service only, you can get away with just a Weekday service configuration. If you operate special service on a day of the week, such as Wednesdays to a market, you could then add a specific service for just those trips that operate on Weekday.

service_id - Required, each service_id should represent one unique service variation such as Weekday Bus, Saturday Trolley, and so on. service_ids may overlap.

Day of week fields are required for monday through sunday but may appear in any order.

start_date and end_date are required, and should be in format YYYYMMDD.

These dates are inclusive and include the cut-off date listed.

#### Calendar Dates

Calendar Dates is optional but is typically used to indicate exceptions to your service information. This file is populated with an individual calendar date, a service type, and if service is being removed (ID 2) or added (ID 1). The exceptional service dates are in year-month-date format (e.g., 20240704). Figure 14 shows an example Calendar Dates worksheet, populated to remove service on holidays.

Figure 14: Calendar Dates Worksheet

service_id – Required, identifies a set of dates when a service exception occurs for one or more routes, should be the same with the service_id from Calendar tab.

date – Required, dates when exceptions occur, the pair of service_id and date may only occur once in calendar_dates.txt.

exception_type – Required, indicates whether the service has been added or removed for the specified dates.

1- service has been added

2- service has been removed

For example, consider an agency that operates a Saturday schedule on Memorial Day or other holidays. In this case:

Calendar Dates would have two records.

Each record lists the date for Memorial Day (date=20240527).

Each record has a different service ID – one for Weekday (service_id=1), the other for Saturday (service_id=2).

The Weekday record would indicate service is removed (exception_type=2).

The Sunday record would indicate that service is being added (exception_type=1).

Calendar Dates can be used to activate or disable service as needed, such as adding special service trips due to additional demand around major events.

#### Feed Info

This file describes your feed itself and may contain similar information as your Agency table and your Calendar table. While optional, this file can help you identify issues with your other GTFS data and can expedite escalation of GTFS data concerns. Figure 15 shows an example Feed Info worksheet that is partially completed and still requires contact information for the feed.

Figure 15: Feed Info Worksheet

feed_publisher_name – Required, the name identifying the publisher of your feed, such as your agency, a consultant, scheduling software provider, or other party who maintains and/or publishes the feed.

feed_publisher_url – Required, the website address for the publisher of your feed.

feed_lang – Required, the language your feed is published in, used to localize data. This can be en, en-US or other language configurations.

feed_start_date – Required, the start date of your feed in YYYYMMDD.

This is similar to your date ranges in Calendar but can help to identify missing periods in your schedule data.

For example, if you have two schedule periods in the same feed it is easy to have the schedule periods overlap in error or have several days missing that should be included.

feed_end_date – Required, the end date of your feed in YYYYMMDD.

feed_contact_email – Required, the contact email for your feed publisher. This should be a point of contact to debug or correct issues with feed data.

#### Routes

Routes is a list of the unique routes operated by your agency with the following fields:

agency_id – Required, the ID of the agency that operates the route, from the Agency worksheet.

route_id – Required, an internal identifier for the route, used to look up the route’s information in other tables such as Trips.

route_short_name – Required, a short identifier for the route that is understood by your agency/riders, such as a route number, or an abbreviation.

route_long_name– Optional, a longer name for the route that describes the service provides, such as Route A – Airport to Hospital.

route_color / route_text_color – Optional, color for the route, such as branding, and a contrast color used for text overlaid on the color – typically you should ensure that color pairs are high contrast, such as white text on a navy route color.

Countinuous_pickup/ continuous_drop_off – whether the rider can board the vehicle at any requested point along the route.

Refer to Representing Deviated Fixed-Route Service for more information.

Pickup and dropoff rules are optional but critical if you provide any sort of deviated service or allow flag stops.

Setting pickup/dropoff rules at the Route level may be overwritten by rules at the Stop Times level.

A sample of route information is shown in Figure 16 below.

Figure 16: Routes Worksheet

#### Trips

Trips lists the specific trips operated by your agency and includes information about the trips. This information includes a trip_id (internal to GTFS), the service_id that the trip operates under (such as Weekday/Saturday), the direction_id to separate trips by direction (outbound or inbound), the shape_id operated by the trip, and optional fields such as headsign.

One core element to understanding the Trips file is that trips do not include the specific time the trip starts or other time information. Times relating to trips are all stored within stop_times.txt.  An example Trips worksheet is shown in Figure 17 below. The yellow columns are part of the Patterns worksheet which is detailed later in this document.

Figure 17: Trips Worksheet

trip_id – Required, a unique internal identifier for the trip, used to look up the trip in Stop Times.

route_id – Required, the corresponding identifier for the route that operates this trip – connects this table to the route in Routes.

trip_headsign – Optional, a headsign for the trip (optional) – this is most useful for loop routes where the headsign may change as the trip progresses in order to identify how far along the trip the loop is.

direction_id – Optional, an ID of 0 (outbound) or 1 (inbound) for the trip.

shape_id – The ID for the specific route alignment referenced in Shapes that this trip operates; one shape may be used across multiple different trips.

This is optional but strongly encouraged, and is required to support any sort of route deviation or flag stops.

wheelchair_accessible – Optional, if the trip supports wheelchairs such as with securements or a lift. Identified using a number of blank/0/1/2.

0 or empty - No accessibility information for the trip.

1 - Vehicle being used on this particular trip can accommodate at least one rider in a wheelchair.

2- No riders in wheelchairs can be accommodated on this trip.

Bikes_allowed – Optional, whether the trip supports allows bikes to be secured or boarded with. Identified using a value of blank/0/1/2.

0 or empty - No information for the trip.

1 Vehicle being used on this particular trip can accommodate at least one bicycle.

2- No bicycles are allowed on this trip.

#### Stops

Stops lists all of your unique stop location information, including the name, location, and descriptions. Figure 18 shows an example Stops worksheet with names, IDs, and location data. There are a number of optional fields available, however the core stops fields include:

stop_id – Required, stop_id can be your passenger facing Stop ID, or an internal tracking ID.

stop_code –  Optional, a supplemental stop code can be used to display a public stop ID for your riders, this is particularly helpful when you have internal tracking IDs for your stops, and a public facing IDs that do not align.

stop_code can be used for more advanced functionality such as connecting platforms to a transit center, or if your stops connect to an outside party and all use a regional ID, while your agency has an internal or agency specific ID.

stop_lat/stop_lon – Required, coordinates for stop locations are identified in separate columns as latitude and longitude. Stop coordinate scan be obtained through a mapping service such as Google Maps. Stop coordinates to the sixth or seventh decimal place are sufficient for high accuracy.

stop_name – Required, the public-facing stop name for the stop.

This should ideally be relatively concise and nicely formatted, such as Main St & Broad St – Farside, rather than a terse truncated name such as MN&BRD_FS.

- Figure 18: Stops Worksheet

#### Patterns

This is not part of the GTFS specification but is added to the workflow to make GTFS creation easier.  Patterns relies on your Trips, Stops, and Routes being entered. An example of an entered Pattern worksheet is in Figure 19.

Figure 19: Patterns Worksheet

Figure 20: Patterns Worksheet

Typically, a pattern refers to a unique route path or service variation, such as serving an employment center during Peak service, and not serving it during periods where demand may be lower or when the facility is not in operation.

For our purposes, pattern is used to refer to unique routing variations and unique running times. Using patterns for unique variations of each makes it easier to maintain your stop times file. The GTFS Creation Workbook contains some automation code to take your unique patterns, combine them with your trips, and create the Stop Times table and sequences based on these values.

Patterns have several columns that are used to either create your Stop Times or assist the user in doing so:

route – A useful route name for the person creating the pattern. This reads from your Routes worksheet but may be hardcoded if need.

direction – This does not correspond to the GTFS direction of 0 or 1 but is instead a list of more common directions such as Inbound, Northbound, Loop, and so on. It may be overwritten if needed and is intended to help in creating the stop sequence and separating patterns.

pattern_id – the ID of the pattern that is entered into your Trips worksheet. This can be a more human readable name such as ‘Blue AM’ or ‘Route 100 Full’ or something concise that makes the pattern easier to follow.

stop_sequence – An increasing integer sequence to order the stops.

stop_id – A stop_id from your Stops worksheet .

time_from_start_minutes_arrival / time_from_start_minutes_departure – the time since the start of the trip that the bus arrives or departs the stop.

Your first stop would likely be 0 for both, as the trip would begin with departing the stop.

The next stops in the sequence would then increase, such as 0, 1, 3, 8; each stop should be later than the next

pickup_type/drop_off_type/continuous_pickup/continuous_drop_off – Refer to Representing Deviated Fixed-Route Service for information on how to configure these columns to represent your service parameters

If empty, these fields indicate passengers are only able to board or alight at specific stops.

Setting these IDs may allow flag stops, or minor route deviations through contacting the agency.

timepoint – If the stop is a timepoint (1) or a regular stop (0). Timepoint locations should have times, while stops are not required to have times.

##### Using Patterns to Create Stop Times

Using patterns takes some planning as well as grouping your service into individual patterns. If you operate a clock-face based schedule where your stop times are generally the same across the service day you may have only one pattern per route. If you have a few service variations where you operate slightly different running times across the service day you may need to make several patterns reflecting these variations.

In each of these cases, grouping your trips into patterns still helps minimize duplicative work you may need to do. Rather than having to create every set of stop times and sequences for every trip, the workbook will populate your stop times based on the existing trips.

Times in the Patterns table are reflected in minutes since the start of the trip for arrival and departure times. This means that if you have a trip with three timepoints, and they have time values of 10 minutes, 15 minutes, and 23 minutes, they would be reflected in a trip starting at 10 AM as:

- Stop A - 10:10 AM

Stop B - 10:15 AM

Stop C - 10:23 AM

A timepoint refers to a specific stop along a route where the vehicle’s arrival time is specified, it’s important for maintaining a reliable schedule. The timepoint field can be set to 0 (approximate) or 1 (exact) to indicate whether the stop times are approximate or strictly adhered to. The exact arrival and departure time should be provided to the timepoints.

##### Configuring Pickup/Dropoff

Another optional field in the patterns tab which is useful for setting up the feed is the pickup/drop_off_type. Some options include '0' for regularly scheduled pickups/drop-offs at the stop, '1' for stops where neither pickups nor drop-offs are permitted, '2' for locations where passengers need to call the agency ahead to arrange their pickup or drop-off, and ‘3’ signifies that passengers must signal or communicate with the driver to request a pickup or drop-off. For example, if one stop is only available for dropping off passengers during specific time periods, then assign 1 to pickup_type, and 0 to drop_off_type.

You can also define the route deviation availability in the Patterns tab by setting up continuous pickup/drop-off, see Representing route deviation service in feed for more information.

#### Stop_times

Stop_times lists the unique arrival/departure times for each service, trip, and stop. If a route operates the same service on Weekday and Saturday service levels then each trip would have its own stop times. Stop times do not explicitly appear as unique as routes or trips, as there is no stop_times_id column. Figure 21 shows an example portion of a Stop Times worksheet. Stop Times is often the largest table in your GTFS, as every stop on every trip is represented with the relevant data, requiring a number of records to represent even a single route’s schedule.

Figure 21: Stop Times Worksheet

Uniqueness is enforced through a combination of the trip_id, service_id, and stop_sequence. The core Stop Times fields include:

trip_id – Required, the trip from trips.txt being referred to, trip_id will repeat in this file as stop_times lists the time for each stop on the trip.

stop_id – Required, the ID from stops.txt that refers to the stop being served.

stop_sequence – Required, the order of stops on the trip, stop_sequence should not repeat on a single trip.

arrival_time/departure_time – Required, these columns indicate the time that the trip arrives or departs at the stop. All stops do not require a specific time, but timepoints (indicated with timepoint=1) on your schedule must have times set. Arrival and departure times may match at a stop but should not have duplicate times on different stops on the same trip.

pickup_type/drop_off_type/continuous_pickup/continuous_drop_off – Refer to Representing Deviated Fixed-Route Service for information on how to configure these columns to represent your service parameters

If empty, these fields indicate passengers are only able to board or alight at specific stops.

Setting these IDs may allow flag stops, or minor route deviations through contacting the agency.

#### Shapes

This file contains geospatial data and functions somewhat like a shapefile or KML. It identifies the point-to-point routing and sequencing for each shape. Shapes identify the alignment followed by each trip. Figure 22 shows an example Shapes worksheet, with a single shape and the coordinate sequence that makes up the route line.

Figure 22: Shapes Worksheet

A single route can have multiple different shape_ids in use, as shapes are tagged to the individual trip rather than the route. Shapes do not have to specifically stop at each stop_id, as long as the shape passes near the stop.

This file is similar to Stop Times in that there is not a single, unique identifier for each record. Uniqueness is enforced through a combination of shape_id and shape_pt_sequence.

shape_id – Required, the ID for the shape – this is repeated across multiple records that make up the same shape.

shape_pt_lat / shape_pt_lon – Required, the latitude and longitude for the point that makes up the shape line.

shape_pt_sequence – Required, the numerical sequence of points, used to arrange them in order to convert into a line.

shape_dist_traveled – Optional distance between each point that makes up the shape; used in some analytical tools to determine operating statistics.

#### Automation utilities

The GTFS Creation Workbook has a few key automation tools built into the workbook. Note that you will need to be able to run Excel workbooks with macros enabled to make use of these utilities. If you are unable to run macros/VBA due to your IT restrictions, you can still create and maintain a functional feed without the automation tools.

Figure 23: Example Utility

#### Using Utilities

There are four core utilities for use in creating your GTFS feed. Each utility is identified in the order in which it would be used when doing general GTFS feed creation/updating. These utilities would be used after you have entered your agency, routes, trips, and stops. You can manually enter the stop times sheet, however a helper utility makes managing this data substantially easier.

Note that these utilities utilize Excel’s VBA extension. This code should work on modern versions of Excel but may require allowing macros to run. Consult with your agency’s IT on how to allow these utilities to run. In general, it is good practice to not run VBA from unfamiliar workbooks (such as one emailed to you), however this section will explain what each utility does which should help to mitigate concerns.

##### KML to Shapes

This utility parses Google Earth KML files that represent your route alignments. Google Earth is available in the browser or as a desktop application. You can also make use of Google MyMaps to create these KML files. Each KML file should be a single route alignment, as one continuous line. Refer to Creating Shapes in Google Earth or Creating Shapes in Google MyMaps for instructions on how to create a line in Google Earth or Google MyMaps.

Figure 24: Example Route AlignmentFigure 24 contains a screenshot of an example route alignment. Each alignment should be drawn to routing for a single route-direction variation. For example, a route that runs between a city and the mall should be drawn separately for each direction, based on the direction of travel. The shape does not need to perfectly snap to the road. Shape data is publicly visible, however, so shapes should be carefully drawn to reflect the path of the route and reflect any unique trip variations.

Figure 24: Example Route Alignment

Figure 25 shows the button to initiate the KML import utility. This utility reads in a single KML file at a time. A window will appear for you to enter the unique shape_id for the KML, and the KML coordinate sequence will be added to the end of any existing shapes data. Note that if you enter a shape_id that is already in use in the shapes worksheet, any existing records will be deleted, and the new data will be inserted. This can allow you to quickly deploy updates due to changes or corrections in your shapes.

Figure 25: Single KML Utility

The shape_id used in this utility should then be entered into the Trips table.

There is also a Bulk KML to Shapes utility. This utility will prompt you to provide a directory of KML files. It will then process each KML file under the same process as KML to Shapes, except it will get the shape_id value from the file name instead of requiring you to manually enter the shape_id.

#### Create Stop Times Using Patterns Table

The Patterns table is not part of the GTFS specification but is included in the GTFS creation workbook as an optional build step. Figure 26 shows the button to initiate the Create Stop Times utility.

Creating and maintaining your Stop Times can be arduous. Every stop arrival/departure on every trip, for all variations in service levels (weekday/Saturday/Sunday) have a unique record in Stop Times. This can quickly become difficult to manage, especially manually. The Patterns table then allows you to abstract out your trips into unique pattern variations.

Patterns in this case refers to any unique variation on a Route + Trip. Variations include changes in stops or stop sequence, shape ID, or even running times. If you are going to use Patterns to create your stop times, you simply need to create each unique Route + Trip variation as a Pattern, and then assign each Trip the Pattern that it is using. The workflow is presented in Figure 27.

Figure 26: Create Stop Times Utility

Figure 27 Patterns to Stop Times Workflow

Patterns include:

The pattern_id – this is unique to the pattern and should not be reused for a pattern that has changed.

The stop_sequence for the pattern_id, in order of stops served.

The stop_id of the stop served on the pattern.

The minutes since trip start for arrival/departure of the stop – arrival and departure time can be the same or may differ in the event of a mid-trip layover.

  - These refer to the total minutes since the start of the trip – your first stop would be 0 minutes, your second stop would be the total minutes to that stop, and then the total minutes to the third stop (e.g., 0, 1, 5)

The minutes since trip start can be blank if you have stop locations that are not timepoints

  - Interim stops on street between transit centers

  - In these cases, set the timepoint value to 0

  - Leave the time minutes value for these stops blank

Note that you will require timepoints to on at least some of the route, as passengers need to be able to determine when trips arrive at a stop. Trip planners, such as Google Transit, will typically assign calculated times to your untimed stops based on how many stops, distance, and time is between each timepoint.

- Other important things to consider include:

Note that the first stop on the trip should generally be set to zero for both arrival and departure.

For stops that are not timed you can omit the time if needed; this would be done if the stop is an interim location that is untimed, or if a specific travel time has not been determined.

Set pickup/dropoff types according to the trip’s route deviation standards.

  - See section Representing route deviation service in feed

Identify if the stop is a timepoint (set to 1) or not (set to 0)

You can then use the Generate Stop Times button to run the generation code. This code will append the Stop Times to the end of any existing data. This can allow you to manually handle Stop Times for some trips, while still generating the bulk of your Stop Times table using patterns. Note that this is an optional step in the event you want or need to manually create your Stop Times.

#### Clear Stop Times

In the event you need to fix and regenerate your Stop Times table, the Clear Stop Times tool can make this process easier. This tool simply deletes all records in the Stop Times table.

This tool will require you to confirm that you wish to delete your existing Stop Times data. Note that this will delete all records, including those you have manually generated.

#### Exporting GTFS

##### Minimal GTFS Export

Note: This will reset any filters active within any sheets during the export process.

This utility will export your GTFS data to the strict GTFS specification. You will be prompted to select a save location in the form of a folder. After you confirm, the sheet will process briefly and export each relevant worksheet to its own file. Some fields, such as the pattern_id in the Trips table will be exported as well. These fields may not be part of the GTFS specification, but do not impact the integrity of your GTFS feed.

You will need to add all GTFS files, with their stock names, to a zip archive. The files should not be zipped within a folder, meaning when you open the zip you should see all the files, rather than needing to navigate into a subdirectory.

##### Full Spreadsheet GTFS Export

Note: This will reset any filters active within any sheets during the export process.

This utility works similar to the minimal export, except it will include the Patterns worksheet and any other columns or sheets you may have added. You will need to select/create the folder that it saves to and add the file to a zip archive prior to using it.

When used in alongside the Full GTFS Import, this allows the export function to serve as a sort of backup, and provides some flexibility to add functionality and information to your GTFS feed over time.

#### Importing GTFS

##### Minimal Import

Note: This will reset any filters active within any sheets during the export process.

This function will prompt you to select a folder using Windows Explorer. It will then search for GTFS files that align with the workbook structure and import data from the GTFS files into the workbook. Data is appended to the end of any existing data, so you may need to manually delete rows with existing data. This can be a useful way to revert or restore your work.

##### Full Import

Note: This will reset any filters active within any sheets during the export process.

When used alongside the Full GTFS Spreadsheet Export, this will allow you to back up iterations of your GTFS feed. This utility will check the directory for any files named after your worksheet tabs (ex: patterns.txt) and import any existing column names. If you are bringing in new files you will need to set up a new worksheet and/or relevant column names for them to be recognized and imported.

## Representing Deviated Fixed-Route Service

The GTFS schema is designed to represent fixed-route transit service, whether bus, rail, trolley, funicular, or ferry. Representing deviating transit service, such as zone-based service or deviated fixed-route service is an evolving aspect of GTFS and is difficult to fully represent the data at the time this document was drafted.

Some issues with representing deviated fixed-route service include:

Zone-based service not represented within many trip planners.

Google Transit does not currently include zone or deviation-based services.

Zones can be included by using the optional GTFS-Flex extension, but this extension has limited support.

Not currently supported by Google Maps or most other trip planners.

Available in some apps, or custom trip planners such as OpenTripPlanner.

Limited support for tools to create and configure zone-based service.

Geographic areas must be created and edited in a separate GIS software.

For example, Google Earth, ArcGIS, QGIS or other geospatial data editors.

Zones are stored as GeoJSON data, a file format and data format that differs from the existing GTFS specification and cannot be edited as a spreadsheet.

As such, there are few good answers to perfectly represent deviated fixed-route service. There are some options to help indicate that deviated fixed-route service is available, and to help passengers coordinate their trip.

#### Introduction to route deviation services

The route deviation is a service that runs along a fixed route, arriving at timing points at scheduled times, but may deviate from the path if a passenger requests a specific pickup or drop-off location within designated zones along the route.

Passengers can request the pickup and drop off service from request stops, or ‘flag stops’, instead of fixed stops. A flag stop refers to a stop where the vehicles “stop on flag”, meaning that the passengers must either signal the drivers or call the agency for arrangement to request a stop, as long as the parking place is safe.

Generally, a route deviation service can be requested by contacting your agency, or by communicating with the driver directly. One important consideration is that if continuous pick-up or drop-off services are allowed, the shape of the route must be accurately defined in the shapes.txt file.

#### Representing route deviation service in feed

You can define the availability of the deviation service in the continuous_pickup/drop_off fields which is set either in Routes or Stop Times.

These fields determine whether the passengers can board/alight the vehicle at any point along the path. A ‘0’ means that continuous pickups/drop-offs are allowed between the regular stops (from this stop_time to the next stop_time in the trip’s stop_sequence), and ‘1’ indicates that they are forbidden. For deviation services that require pre-arrangement through your agency, use a '2' in these fields. For services where passengers must coordinate directly with the driver, assign a '3'.

Pickup and dropoff settings can be applied using the columns in Routes and will apply to all trips and stops on that route.

You can also apply pickup and dropoff settings individual stop occurrences via Stop Times.

If pickup/dropoff settings are present in both Routes and Stop Times, the Stop Times settings will override the setting in Routes for that stop occurrence.

##### Configuring Route Deviation

Here are some scenarios and instructions for setting up the GTFS feed for your deviation services:

Default scenario: Vehicles only stop at specific locations (indicated in the stops.txt file) during routine operations. Vehicles do not stop at stops they are not schedule for, such as locations where the vehicle would be unable to safely serve or that are not on the route’s alignment.

In this case, pickup_type and drop_off_type can both be set to 0 or left empty.

If passengers can board or alight between stops, then continuous_pickup and continuous_drop_off would both be set to 0. 0 indicates that flag stops are allowed for either pickup or dropoff, and the value must be set to 0.

Scenario 1: Pickups at flag stops with prior arrangement through the agency are allowed. Passengers can phone the agency to arrange pickups within ¾ of a mile off fixed route. Passengers need to notify the driver if they want to alight the bus between two scheduled stops along the route or within several blocks from the route.

In this case, the continuous_pickup is set to 2, and the continuous_drop_off is set to 0.

Scenario 2: Pickups at flag stops with prior arrangement through the agency are allowed. Passengers need to coordinate with the driver to request a drop-off that deviates from the route by more than ¾ mile.

In this case, the continuous_pickup is set to 2, the continuous_drop_off is set to 3.

Scenario 3: Service is restricted solely to regular scheduled stops. Passengers cannot board or alight from the vehicle at any point along the travel path.

In this case, both the continuous_pickup and continuous_drop_off are set to 1.

Scenarios 4: Special service at specific time of day:

E.g., route A is scheduled to pick up passengers from Walmart at 3 PM and transport them directly to their destinations without dropping off people at Walmart or following regular fixed stops. In this scenario, the pickup_type at Walmart would be set to 0, signifying that only pickups are permitted there. The drop_off_type would be set to 1, indicating no drop-offs at Walmart. Additionally, continuous_pickup will be set to 1, and continuous_drop_off will be set to 3. This indicates that passengers will be dropped off at their individual locations as coordinated with the driver, and no further pickups will occur during this run.

# Editing a Feed

While this spreadsheet allows users to create a GTFS feed from scratch, it’s also designed to maintain a feeds over time whether created in the spreadsheet or imported from an existing static GTFS file. This section details how to make various edits that are typical when implementing a service change.

The following sections will detail some routine data operations, and things to consider when making edits to your GTFS feed. GTFS schedule elements have relationships that must be considered when editing a feed or else you may quickly make your data invalid.

Some steps may require working around Excel data validation restrictions. If you need to do bulk edits, such as deleting multiple entire rows – and are comfortable with disabling worksheet integrity protections – you can disable worksheet protections as detailed in the Worksheet section. It is strongly encouraged to reenable protections whenever possible, as these sheet restrictions ensure that required columns stay present, and column ordering is important to some utility functions.

## Importing an Existing Feed

The Workbook has two utilities to import an existing feed, which may be necessary if you need to make edits to reflect a service change. The two utilities work in similar ways:

- Click the utility button to run them.

Navigate to the folder with your unzipped GTFS files.

All files (such as routes.txt and trips.txt) must be in the same directory, and not in a zip archive.

The utility will then search for any .txt files that match worksheet names.

The utility will warn about any missing files that are expected, but you can either manually enter the data later or determine if it is optional.

The imported data will then overwrite the spreadsheet contents where applicable.

The specific differences between each utility are outlined in the Importing GTFS section. If needed, you can add new columns and worksheets to existing data, but you will need to use a specific import utility to get new data types. Note that you may need to disable sheet-level protections, as detailed in Worksheet .

If you need to import files not listed in the GTFS Workbook you can add the file as a new tab, with a matching name for the file (such as “fares”), add any required columns to the new worksheet, and then run the full GTFS importer.

Note that importing a feed will not create patterns out of your existing trips and stop times.

## Routine Data Updates

The following sections will detail high-level steps to perform routine GTFS data updates, as well as the cascading impacts of making changes to one aspect of your GTFS data.

### Adding or Deleting Routes

- Navigate to the Routes worksheet

Identify the route to be added, updated, or deleted

If deleting a route, delete each cell’s values (you may be unable to delete an entire row without disabling sheet protections)

If you are adding a new route, you will need to add the route to the Routes worksheet and then add trips, and determine if any new stops or patterns are required

If you are simply changing a route’s name or branding, you will not need to make further changes unless you have changed the route_id field

Enter or update the required and optional values for the route, including the ID, name, and color branding

### Adding or Deleting Stops

- Navigate to the Stops worksheet

Identify the stop(s) to be added, deleted or updated

If deleting a stop, delete the value in each cell (you may be unable to delete an entire row without disabling sheet protections)

If you are simply changing a stop’s name, making minor latitude/longitude corrections, or adding a new stop_code, you likely do not need to make any further updates

If you are adding one or more new stops, you will need to add the stops to the Stops and the Patterns (or Stop Times) worksheet, and determine if any new stops or patterns are required (such as if a stop is only serviced on specific trips)

Refer to Using Patterns to Create Stop Times for more detailed information on using the Patterns sheet

Navigate to the impacted pattern(s) in the Pattern worksheet (this process is similar for Stop Times as well)

Edit the stop sequence, inserting the stop where appropriate – note that you may need to disable sheet protections, sort the sheet, or drag values around using the Excel interface

Enter the new stop times, using the stop_id and stop_sequence to ensure that the correct stop is in the correct order

If you are using patterns, run the Clear Stop Times utility to remove your old Stop Times data, then run the Create Stop Times utility to generate new stop times

### Editing Calendar

The Calendar worksheet consists of IDs for each service type, with a 1 if the service operates on the particular day located in the column heading. This workflow will highlight how to modify an existing service, or to set up a new service, with an overview of potential impacts on your trips.

##### Splitting an Existing Service

In this example, we will be changing a Monday-Friday service into Monday-Thursday and a special Friday service. This would be a case where you need to run reduced service on a specific day or are implementing weekend service.

- Navigate to the Calendar tab, and identify the specific Service to be edited

- Change the days that service will not be operating to 0, and leave any days service does operate to 1

- Create a new service for Friday, with its own service_id. Set all days to 0 except for Friday

- Determine what trips will operate on Friday – it may be that you run almost all trips on this Friday service, except a handful of routes

- Create those trips that will run on Friday

  - You can copy the existing trips from the Monday to Thursday service ID, but you must give all of those trips unique IDs(ex: Route_A_Wkdy_1 becomes Route_A_Fri_1)

  - Set the service_id for those trips to your Friday service ID

##### Layering Multiple Service types

You can assign multiple service types to a single day. For example, your agency may be running special service every Friday, rather than less service. In this case, you have two options: using scheduled service in the Calendar worksheet, or using exceptions in Calendar Dates.

The calendar process is best for cases where your service runs every type of day that is in a day range, such as special Friday service when a university is in session, or a shopper shuttle during the holiday season.

###### Calendar Process

- Create a new service ID in Calendar, with 0 for days the service does not operate, and 1s for the days it does. Set the dates that this service should start and end.

  - Note that you may have multiple services operating on the same day, it is only the ID for each service that must be unique

- Create new trips to use the new service ID. These trips are expected to run every day of the week that is set in addition to other trips with service IDs set to run on those days.

###### Calendar Dates/Exceptions Process

This process is best for services that do not operate on the same day of the week within a date range. This might be special trips during a festival, or reduced or special service on a holiday.

- Create a new service in Calendar, with all 0’s for each day.

  - Note that this is not technically GTFS best practice but is used in the Workbook to help with some data validation and to populate dropdowns.

- Add a record in Calendar Dates for each date that this service would run.

  - Set the exception_type field to 1 for dates this service is being added

  - If other service is being removed on these days, you would add another record that that service’s ID and set the exception to 2, for removed service

  - If you are only adding service to these days you may leave the existing service in place, effectively layering service together.

- Create new trips to use the new service ID. These trips are expected to run every day of the week that is set.

### Editing Trips and Patterns

Editing a trip can typically done most easily by using the Patterns functionality built into the workbook. Refer to Using Patterns to Create Stop Times for more information on how to create and use patterns.

Generally, you will need to make a new pattern or update an existing one if you have made changes to either the sequence of stops on a trip, or to the running times between stops or for the trip overall.

If you have added either a new trip type, made changes to your stop sequences, or need to incorporate a new stop you would:

- Navigate to the Patterns worksheet

Find the pattern to be edited

Determine if you either need to add a new stop, change the time assigned to one or more stops, or create a new pattern entirely

If updating or adding a stop, enter the time that the stop is served, in minutes, since the trip started

For example, the first stop on the trip would typically be 0 minutes, while the following stops would be an increasing number of minutes

Example: 0 minutes, 1 minute, 2 minutes, 5 minutes, with the last stop being served at 55 minutes for a trip that takes 55 minutes total

You can remove a stop by removing the values, and then either deleting the row (requires unprotecting the sheet), sorting the sheet, or dragging data upwards

If you created a new pattern, you may need to update the stops impacted. Within the Trips worksheet, you can easily assign the pattern to new or existing trips using the dropdown, or by typing the pattern’s ID

### Clearing Stop Times

Clearing stop times only needs to be done if you have made edits to your patterns and would like to regenerate your stop times. This is done using the Clearing Stop Times utility.

You only need to clear and recreate your stop timing if you have made changes to your patterns, in which case you would need to rerun the Create Stop Times Using Patterns Table utility. If you manually edit and create your stop timings, you generally will not want to clear this data out.

### Adding or Replacing a Shape

Shape edits should be performed on individual KML files, as outlined in Creating Shapes in Google Earth. Shapes must be stored with one route alignment in each individual KML file, meaning that each file must be a single, contiguous line that represents only one route-direction and alignment.

- You can import new shapes data using the KML to Shapes utility. This utility will prompt you for a KML to import, and an ID to assign to that shape.

- If the ID is already in use, it will overwrite the existing shape in your Shapes workbook.

- If the shape is new, you will need to add/update any relevant trips that make use of the shape.

# Feed Validation and Debugging

#### Feed Validation

Ensuring that your GTFS feed is valid is a crucial step of the building process. You can validate your GTFS feed using a validator, and skipping validation may result in errors being found after publishing the feed, when your schedule may already be live. Errors at this stage may impact passenger information, your ability to provide quality customer service, and trip planning.

Validation, in this case, does not refer to whether or not your GTFS data perfectly reflects your schedule, though that is one-way errors are introduced. Validation refers to reviewing that your GTFS package is valid under the specification and can identify schedule irregularities such as vehicles traveling too quickly, improper stop placement and more.

We’ll review how to validate a GTFS file using Foursquare ITP’s Reflect validator [https://reflect.foursquareitp.com/]. Reflect is free and available for all agencies to use.

To validate a feed:

Zip your feed files (files must be zipped by themselves, not in a directory and then zipped)

Ex: transit_gtfs.zip contains only text files, not a directory called my_gtfs that has the files within it.

Upload the file to Reflect and enter your email address.

Review the results.

Results will appear as a count of the High, Medium, and Low severity issues, and a spreadsheet of each issue will be emailed to you.

High severity issues are ones that may lead to your feed being rejected by processors such as Google Transit or Apple Maps. These are typically issues that make your feed invalid, such as duplicate records that should be unique, or missing required records.

Medium severity issues may reflect data that is incorrect, such as vehicles scheduled to travel at impossible speeds or too quickly. These are issues that should be corrected but are not likely to impact overall feed validity. These issues can highlight data that was entered incorrectly.

Low severity issues are typically related to feed aesthetics. This may include route or stop names, or issues that may impact the passenger experience.

#### Understanding GTFS Errors

The spreadsheet sent by Reflect contains several columns:

error_id – the unique ID of the error

priority – The severity of the error in Low/Medium/High

error_name – A brief classification of the error.

error_message – A message describing the error.

entity_type – The file that the error was found in.

entity_id – The ID of the record that caused the error.

Ex: stop_id, route_id, trip_id.

line_number – The line number within the file that the error was found.

This is the line number + header, so line_number=2 would be the first row of actual data.

bad_value – The value that caused the error.

related_routes – Routes impacted by the error.

Errors may not actually be an issue if they are intentional or are low/medium. For example, you may receive errors for missing service on days within your GTFS service date range. This may be intentional, such as if your agency does not operate Sunday or some sort of Holiday service.

Other errors, such as Low severity errors for route names, typically will not impact feed validity. Fixing errors from aesthetic causes are good practice, however, as it can improve your rider experience when using services such as Google Maps or other trip planners. For example, a route with branding color and text that are low contrast may make your information appear illegibly in trip planners.

#### Error Examples

This section will introduce a common error, and a common error type (referential integrity). Referential integrity is a term for when your GTFS structure has an invalid relationship, typically due to either missing or duplicate IDs in one or more GTFS files. For more on referential integrity refer to Referential Integrity. Referential integrity is core to the validity of your GTFS feed, and understanding how to quickly correct missing or duplicative data can make resolving these issues much easier.

##### Stop Placement

For example, you may have a Medium Priority error of “Stop outside geographic area” for a stop that is very far away from the majority of the stops in the feed. Because this is a Medium error, it may not be required to fix. This error may also be acceptable, as in the case of commuter routes that have stops very far away from an agency’s core service area.

You can trace this error by reviewing the entity_id (in this case it would correspond to the stop_id) and confirm where the error is arising in the stops.txt file. You can then trace the line_number in the Data Creation workbook. Since Excel has line numbers built into it, you would just need to track down the line number or the stop_id and confirm if the stop is placed correctly or not.

##### Referential Integrity

Another common error type is broken referential integrity. This essentially means that two records (such as a trip in trips.txt and a route in routes.txt) are unable to be connected. Some of the most common causes of this are a missing ID in one table, duplicate stop times, or a duplicated ID.

In the case of a missing ID, you would need to determine which file needs to have a record added, or if one needs to be corrected. You may have trips that refer to a route_id 1O5 – in this case, you may find that you meant to enter one-zero-five, but that the route was incorrectly entered as one-letter O-five in the trip. This could be fixed by correcting the typo.

Occasionally, issues arise with stop times. Generally, one would expect the stop arrival and departure times to increase as the sequence number for a trip increases. However, there are instances where the stop times between two consecutive stops are identical, or a stop with a smaller sequence number has a later stop time. The simplest method to rectify this error is to examine the Patterns table in the worksheet and checking the time from start minutes fields.

Alternatively, you may have a trip that reflects to duplicate route_ids. Trips would refer to a single route_id of 105, but within your routes.txt, two records may have route_id 105. Identifying which one is the correct route can fix this issue.

#### GTFS Worksheet Debugging

You may also find that something went awry in the GTFS Creation Workbook. This could be that data was imported from a feed in a strange way, or some other connection issue. This section outlines some common issues and how to address them.

##### GTFS Files exported with empty fields

In this case, check to see if there are any empty records in the individual workbook tabs. The GTFS Export utility will trim empty rows from the end of your workbook but will not delete empty records in the middle of your files. Other utilities that import data may append data to the end of your existing data; if the file had data in a random cell, this could result in data existing out of your view pane.

##### Incorrect Calculations/Broken Utilities

Check that new columns were not inserted between existing columns, or that columns were not reordered. Some of the utility code depends on specific column positions, and reordering columns may break the processing code.

New columns, if needed for your GTFS static feed, can be added at the end of existing columns (to the right). This way, they will be included in your export but will not impact GTFS processing.

##### Invalid format errors from GTFS Validation

Excel will handle data transformations on the fly. This can be helpful when it works intuitively but can also result in data not existing in a format you expect. For example, if you have a route called 1-12, Excel may understand this as a Date and change the data type, resulting in issues when exporting.

To fix this:

Left click the column (ex: A, B) to select the entire column, or the specific record.

Right-click the column/cell and select Format or use the Format dropdown under Home.

Change the Format to Text/Number as required.

This can help address issues depending on your route numbering schema. For example, some agencies will identify their routes as three-digit numbers, padded with zeroes such as 001, 003, 099, 150. Excel will often transform any left-side zeroes into a number, resulting in routes being renumbered to 1, 3, 99, 150. This can result in referential integrity issues.

# Hosting a GTFS Feed

#### Hosting Needs

GTFS feeds must be hosted at a static and unchanging link. This can be more challenging that it may appear, as many Content Management Systems such as WordPress or Drupal will put new file uploads into a specific file directory that changes. This means that your link may change from youragency.org/month/day/gtfs.zip to youragency.org/monthplus1/day/gtfs.zip, making your GTFS unable to be retrieved.

GTFS consumers such as Google Maps will periodically download your static GTFS data from your link. If the link itself has changed, even if it is linked to from the same page, your GTFS cannot be retrieved.

Some options for hosting a file content URL that does not change are:

You or your webmaster manually upload or overwrite the feed contents using a service such as National RTAP’s feed hosting.

You may also be able to use cloud storage, but you must be able to ensure that the link does not change over time or when updating your zip file.

Google Drive may be an option for file storage.

Note that this process may change if Google updates Google Drive, and it is not recommended to be relied upon for long-term storage.

You will need to create a Google account.

You will need to upload the zip file and make it publicly readable.

Refer to Hosting Files on Google Drive for guidance on how you can replace the file and get a static URL.

You may also be able to manually upload your feed to a distribution service such as Mobility Data or Transit.land.

FTA requirements at the time that this document was drafted require providing a direct link within the FTA NTD dashboard. FTA currently allows a direct upload but requires explaining why you cannot provide a file URL.

#### File Structure

Your GTFS feed must be hosted as a .zip file, with the individual .txt files immediately within the .zip. They should not be in a directory within the .zip. All files should have file names per the GTFS specification, such as agency.txt, rather than agency_v2.txt.

Your GTFS feed should be structured as a comma-separated value (CSV) file with a .txt file extension. Feeds should use commas to separate columns in data, instead of other formats such as tab-separated or standardized column widths.

Avoid padding data with spaces, 0’s or other characters whenever possible. This can lead to processing issues.

#### Validation

You should use a feed validator prior to hosting any feed for use. Validating your feed is critical. Ideally a validator would return zero errors, but it is important to allow time to run a validator, identify errors, and fix errors. GTFS errors may identify issues in your overall schedule or may just be data integrity.

Refer to Feed Validation for more information.

#### Overlapping Feeds/Services

In the case of service changes, a single feed can be created with overlapping service information. This service information must be disjoint, such as using a Weekday_1 and Weekday_2 service IDs where one begins service the day after the other ends. This can be complex and require duplicative data, such as listing the same route with different GTFS IDs to represent differences in its services across service changes.

When creating an overlapping feed to address a service change you would need to duplicate trips that are unchanged between dates and ensure that the changed data has all relevant updates such as stop sequences, patterns, shapes, and so on.

# A. Appendix

## Data Validation and Restrictions

The spreadsheet tool is configured with several help texts and restrictions to aid development of the GTFS feed.

### Help Text

The column header of each row can be selected to display help-text on the contents of the column. This help text can aid in understanding the expected data format, requirements for the data, or other details. For example, dates in your GTFS feed are stored in YYYYMMDD format. Columns that contain date data will reference this format when the column heading is selected.

### Data restrictions and dropdowns

When possible, this sheet makes extensive use data entry restrictions and dropdowns. The sheet expects that you are working from left to right, or macro to micro scale, such as starting with your Agency, your Routes, and all the way to individual stop times on each trip. While this macro-to-micro workflow generally holds true, you will not always make updates in this format, or some data may be more iterative.

To assist in making updates or working iteratively, lower-level sheets read data from higher-level sheets. For example, your Stop Times has restrictions and dropdowns to only allow existing IDs from your Stops sheet. This helps to prevent data entry errors and can indicate that a stop is missing if you are unable to create a stop time.

Dropdowns work by either reading from a hidden Dropdowns worksheet, or by reading from your existing data. If you are unable to enter data, it’s best to check your existing data first before checking the hidden Dropdowns.

In the event that you enter invalid data for a cell, an error message will appear that describes the data validation error that occurred.

### Worksheet Protections

Each data entry worksheet in the file is locked. Generally, the lock is minimal, and only applies to adding, deleting, or formatting columns. GTFS data is highly dependent on column names, and some processing either relies on specific columns being present, or in a specific order.

If you need to add new columns to your data, or modify column formatting, you can unlock the worksheet. It is highly recommended that you back up your sheet before doing so as once the sheet is unlocked some column data may not function as anticipated.

To unlock the sheet:

Navigate to Review in the top ribbon.

Click Unprotect Sheet.

The sheet is now unprotected.

You should lock the sheet again after you are finished. You can easily lock the sheet again and keep the same restrictions by:

Navigate to Review in the top ribbon.

Click Protect Sheet.

Accept the parameters that allow/restrict settings and leave the Password field blank.

## Excel Tips

The entire GTFS Creation Workbook is based in Excel. There are some useful shortcuts and features that may help when working in Excel.

### Find and Replace

You can quickly Find relevant information pressing Ctrl+F, or by clicking the Find button on the Home tab (Figure 28).

Finding information lets you search for whatever the text you have entered is. For example, maybe you  want to search for a specific route in your Trips tab. Using Find would let you search for all instances of that route, while Replacing the information allows you to enter a search text (such as Route A), and then replace all instances of it with another text (such as Route A – AM).

When Finding or Replacing text, you can specify search parameters (Figure 29). Some useful options are:

- Within Sheet or Workbook – Limit your find/replace to just the active sheet, or to your entire Workbook file

- Look In: Formulas to replace the contents of formula (such as the range of cells a formula applies to), or to Values to update values only

- Match Case - Require that the search match capitalization exactly, such as “Trips” being different from “trips”

- Match Entire Cell Contents – The full contents of the cell must match the search text, meaning that searching for “Trip A” will not affect “Trip AB”

Figure 28: Using the Find Icon from the Home Ribbon

Figure 29: Configuring Replace

Filter and Sort

The workbook comes configured with filters on each worksheet, and sorting options.

Sorting should not impact the integrity of your GTFS file, but may make it difficult to follow your data, such as if Stop Times is not sorted by Trip, then Stop Sequence. Filtering and sorting use a similar interface. Simply click the arrow next to each column name to show the options to Filter or Sort. See Error! Reference source not found. for an example of the Filter/Sort dropdown.

##### Filtering Data

When filtering data, you can enter text to search for values to filter on. Then simply click the checkbox next to each to toggle the data visibility on or off. Note that your data is still present, the row itself is just hidden. This may lead to issues entering data in subsequent rows, as there may be hidden rows between the data you have filtered to, and the data you have added.

You can filter for virtually any value you have, or even filter for cells formatted in a specific way, such as highlighted or using other Excel functions such as Conditional Formatting.

Figure 30: Filtering/Sorting Data

##### Sorting Data

Data can be Sorted using the same dropdown as filtering data. You can easily sort A-Z, Z-A, or sort numerical data by largest/smallest.

You can sort on multiple columns using the Custom Sort functionality from the Home Ribbon > Sort menu. Note that this is an advanced functionality and is disabled due to sheet restrictions. Should you have a need to use a custom sort, simply turn off the restrictions under Review > Unprotect Sheet. You can then use Custom Sort to sort by multiple columns. Note that columns are sorted based on a hierarchy. For example, if you needed to sort your Trips to make it easier to follow after substantial editing you would sort by:

Service type to group all Weekday/Saturday/Sunday trips together

Route ID to group all related routes together

Trip ID to order based on your Trip ID schema

## Creating Shapes in Google Earth

This section outlines how to create shapes in Google Earth. The Desktop version of Google Earth is shown, but the process is similar for the web-based version. Google Earth can be installed by your agency IT. It may require administrative rights.

A fresh Google Earth session will start at a global scale. Search for your service area county to zoom in. Figure 31 shows the Google Earth App UI. Under the Places bar, we can begin adding a new Path. A Path is the Google Earth term for a LineString or Line. Our goal is to draw a single continuous Path for each unique route travel path.

Figure 31: Google Earth Desktop UI

As Figure 32 shows, you can create a Path by either going to Places, right-clicking and selecting Add > Path, or by clicking the Path icon in the top bar.

Figure 32: Creating a Path Two Ways

A New Path window will appear as in Figure 33. Within this window you can name your path with a descriptive name such as the shape_id, or a brief description of the variation such as Route A – Northbound. You will then want to leave this window open – you can move it so it is not covering the map pane – and begin clicking on the map to draw your route.

Some notes on controls:

Left-clicking will draw points in a sequence – when you are finished simply click OK in the path window.

You can pan the map using arrow keys, or by using the W, A, S, or D keys on your keyboard.

W will pan up, A pans left, S pans downward, and D pans right.

You can delete the last point by right-clicking, or holding Delete will rapidly delete points.

You can draw curves like clicking and holding left-click, and slowly dragging along a highway.

This is likely overly detailed, and it may be easier to manually click a few points.

Under Style/Color in the path window you can color the path, allowing you to visually delineate between paths – this may help you maintain your paths over time.

The default path color is White with a 1-pixel width, so changing this to something easier to see can make path creation substantially easier.

You can left click and drag any existing point to update or fix a line.

Figure 33: New Path Window

#### Editing Existing Lines

You can edit any existing line by right clicking the layer, and selecting Properties. This will immediately drop you into edit mode. From there, you can extend the line, or use the controls above to edit your line.

#### Exporting Lines

Each line should be exported individually. You can right-click the specific layer, and go to Save Place As. Your Line will need to be saved as a KML, as shown in Figure 34, rather than a KMZ. The default file extension is KMZ, however this is a different file format and not able to be parsed by the KML utility.

Figure 34: Changing Google Earth Export Format to KML

## Creating Shapes in Google MyMaps

Google MyMaps has a similar workflow to Google Earth. You will need a Google account to use this. Working in Google Maps divides your work into projects. You can start a new project by clicking Create a New Map.

Within the layer pane you can create, import, and manage layers.

If you have existing KML layers, you can Import them into a Layer as shown in Figure 35, or you can Create a layer. Layers can also be renamed (along with your map project) by using the three horizontal dot menus.  You can also customize the Base map from the Google vector tile layer to a different base map such as a satellite view. This can help when drawing lines and comparing against real world conditions such as private roads.

Figure 35: Google MyMaps Layer Pane

You can also have multiple layers per project. In Figure 36 below there are two separate layers – Layer 1, and Layer 2. These have been renamed. When drawing lines, your line will be drawn and added to the active layer. You can identify the active layer by which layer has the blue accent to the left side.

Figure 36: Active Layer Example

After you’ve selected the appropriate active layer, you can begin drawing. In Figure 37 below, you can click the Line icon and selected Add line or shape. You can draw other line types, but they may be restricted to paths and not behave as expected.

Figure 37: Drawing a Line in MyMaps

As you begin drawing a line, each point will appear as an opaque circle on the line as in Figure 38. You cannot edit the line while drawing. You can terminate the line by double-clicking the final point. You will then be given a prompt to name the line. Name it and click OK. Figure 39 presents the naming pane.

You can then edit the line by dragging the opaque points. There are also transparent points – these are the midpoint between line segments. If you click and drag these points you will break the line and split it into new segments. This can allow you to make detailed edits to your line very quickly.

Figure 38: Drawing a Line

Figure 39: Naming a Line

#### Editing an Existing Line

To edit an existing line (either imported into MyMaps or one previously created) simply double-click the Line Name under the Layer as in Figure 40. The line should activate, and you will be able to see the vertexes that make up the line.

You can then drag existing points or drag the midpoint of each segment to add additional points. If you need to extend a line, any clicks on the map will extend the end of the line. If you need to extend the start of the line, however, dragging the midpoint to add a new vertex and then dragging the start point is generally the easiest way to do that.

You can delete any existing point by right-clicking it and selecting Delete Point as in Figure 41.

Figure 40: Editing an Existing Line

Figure 41: Deleting a Point

#### Exporting a Line

After your line is completed, it will be exported, one route-shape at a time. Click the three-dot menu on Layer that contains your single continuous line. Go to Export Data and select KML/KMZ as shown in Figure 42.

A window with export settings will appear as in Figure 43.

Ensure that you are setting it to export to KML instead of KMZ, and make sure the “keep data up to date…” box is unchecked.

Save the KML file to your target directory for further processing.

Figure 42: Exporting to KML

Figure 43: KML Export Settings

## Hosting Files on Google Drive

Google Drive can be used as a file host if your agency does not have the ability to upload a file to a static URL. A static URL is one that does not change over time. For example, you should be able to overwrite the contents of your.agency/gtfs.zip with a new GTFS zip file, and have the URL return the new file.

GTFS consumers such as Apple Maps, Google Maps, and other apps, will retrieve the contents of this URL. The URL needs to return the zip file itself and must be unchanging over time.

Google Drive is one such option for hosting a static feed. Note that this functionality may change over time, so it is recommended to only use Google Drive while working to identify alternative hosting.

- Create a Google account – you can use an existing Gmail account.

This account could also be used to help your agency become a Google Transit Partner which gives you access to the Google Transit Partner Dashboard – the dashboard provides enhanced GTFS feed capabilities and also allows publishing service alerts.

Visit drive.google.com.

Click New and upload your GTFS zip file.

Find your file within Google Drive and click Share.

Change the file permissions so Anyone with the Link can View the file (Figure 44).

Copy the link and save the text somewhere – we will need to modify it slightly to share with consumers.

By default, Google Drive shares links with options to see the contents – we need a link that automatically downloads a zip file.

Your link will look like https://drive.google.com/file/d/randomCharacterAndNumbers-Bj6/view

Copy the bolded text between /d/ and /view – this is the file ID.

Paste the file ID into the URL below, replacing the bolded text in the bullet below.

https://drive.usercontent.google.com/uc?id=TheIDyouCollected&export=download

Verify that the full URL is shareable and initiates a zip file download when visited.

Figure 44: Sharing Permissions to View File

You can also overwrite the contents of a file hosted on Google Drive. Overwriting must be done carefully to ensure that the file URL does not change.

- Within My Drive, left click to select the GTFS zip file.

Click the three-dots next to the file to expand the menu.

Navigate to File information > Manage versions and left click (Figure 45).

Click Upload a New Version and upload your new GTFS feed.

You will see that a new version has overwritten the previous file contents.

Your file names do not need to be identical between versions.

The newly uploaded file will be renamed within Google Drive.

Verify that the URL has not changed from previously collected URLs.

Figure 45: Navigating to Manage Versions

See: https://en.wikipedia.org/wiki/List_of_tz_database_time_zones for a list of allowable time zones under “TZ Identifier”